Module 1

Assessment Questions:

## 1. In which OSI layer does the Wi-Fi standard/protocol fit?

Wi-Fi operates in the lower layers of the OSI (Open Systems Interconnection) model, specifically in Layer 1 (Physical Layer) and Layer 2 (Data Link Layer).

### Layer 1 - Physical Layer

The Physical Layer is responsible for the actual transmission of data over the air using radio signals.

- Wi-Fi uses radio frequencies such as 2.4 GHz, 5 GHz, and 6 GHz

- It defines how bits are converted into signals and transmitted

- It includes modulation techniques like OFDM (Orthogonal Frequency Division Multiplexing)

- It handles signal strength, frequency channels, and data rates

Example: When you connect your phone to Wi-Fi, the signals traveling between your phone and router happen at the Physical Layer.

### Layer 2 - Data Link Layer

The Data Link Layer ensures reliable data transfer between devices.

It has two sublayers:

- MAC (Media Access Control) Layer

- Handles device addressing using MAC addresses

- Controls how devices access the wireless medium using CSMA/CA (Collision Avoidance)

- Manages frame transmission and retransmission

- LLC (Logical Link Control) Layer

- Provides error detection and flow control

- Ensures proper communication between devices

Example: When multiple devices (laptop, mobile, TV) are connected to the same Wi-Fi, this layer ensures they communicate without collision.

### Standards Used

Wi-Fi is based on standards developed by IEEE under the 802.11 family.

These standards define:

- Frequency bands

- Data rates

- Transmission techniques

- Security mechanisms

### Conclusion

Wi-Fi mainly operates in:

- Layer 1 (Physical Layer) - Signal transmission

- Layer 2 (Data Link Layer) - Data framing and communication control

## 2. Wi-Fi devices used in daily life and their wireless capabilities

In daily life, we use multiple Wi-Fi-enabled devices. Each device supports a specific Wi-Fi standard (generation), which determines its speed, frequency, and performance.

### Common Wi-Fi Devices

- Smartphones

- Laptops

- Smart TVs

- Tablets

- IoT devices (smart speakers, cameras)

### Wi-Fi Generations and Standards

Wi-Fi technology has evolved through different generations:

| Generation | Standard | Frequency | Speed | Features |
| --- | --- | --- | --- | --- |
| Wi-Fi 4 | 802.11n | 2.4/5 GHz | Up to 600 Mbps | MIMO support |
| Wi-Fi 5 | 802.11ac | 5 GHz | Up to 3.5 Gbps | Faster speed, beamforming |
| Wi-Fi 6 | 802.11ax | 2.4/5 GHz | Up to 9.6 Gbps | OFDMA, better efficiency |

These standards are defined by IEEE and certified by Wi-Fi Alliance.

### Example Mapping of Devices

#### 1. Smartphone

- Modern phones support Wi-Fi 5 or Wi-Fi 6

- Properties:

  - Dual-band (2.4 GHz & 5 GHz)

  - High speed

  - Better battery efficiency

#### 2. Laptop

- Usually supports Wi-Fi 5 or Wi-Fi 6

- Properties:

  - Higher bandwidth usage

  - Stable connection

  - Supports advanced features like MU-MIMO

#### 3. Smart TV

- Mostly supports Wi-Fi 4 or Wi-Fi 5

- Properties:

  - Used for streaming

  - Requires stable connection more than high speed

#### 4. IoT Devices (e.g., Alexa, CCTV)

- Mostly use Wi-Fi 4 (802.11n)

- Properties:

  - Operate on 2.4 GHz

  - Low power consumption

  - Lower data requirements

### How to Check Device Capability In Real Time (Practical)

We can check your device’s Wi-Fi properties by:

- Opening Wi-Fi settings

- Viewing connection details

- Checking:

  - Frequency band (2.4 GHz / 5 GHz)

  - Link speed (Mbps)

  - Protocol (802.11n/ac/ax)

### Conclusion

Different devices support different Wi-Fi generations based on:

- Usage (streaming, browsing, IoT)

- Speed requirements

- Power consumption

Modern devices use newer standards for better speed and efficiency, while older or IoT devices use simpler standards for stability and low power usage.

## 3. What is BSS and ESS?

In Wi-Fi networking, BSS (Basic Service Set) and ESS (Extended Service Set) are fundamental concepts that describe how wireless networks are organized.

### Basic Service Set (BSS)

A BSS is the smallest unit of a Wi-Fi network, consisting of:

- One Access Point (AP)

- Multiple wireless devices (stations)

All devices communicate through the AP.

#### Key Features:

- Uses a single AP

- Has a unique identifier called BSSID (MAC address of AP)

- Covers a limited geographical area

- All communication is centralized via the AP

#### Example:

A home Wi-Fi network where:

- One router provides internet

- Devices like phone, laptop, and TV connect to it

This is a Single BSS network

### Types of BSS

- Infrastructure BSS

  - Uses an Access Point

  - Most common type

  - Example: Home, office Wi-Fi

- Independent BSS (IBSS)

  - No Access Point

  - Device-to-device communication (Ad-hoc mode)

### Extended Service Set (ESS)

An ESS is formed by connecting multiple BSS networks together.

- Multiple Access Points are connected through a wired network (like LAN)

- All APs share the same SSID (Wi-Fi name)

- Allows seamless roaming

#### Key Features:

- Large coverage area

- Multiple APs

- Same network name (SSID)

- Supports mobility (roaming)

#### Example:

College or office Wi-Fi:

- Multiple routers in different areas

- Same Wi-Fi name

- User moves without disconnecting

### Difference between BSS and ESS

| Feature | BSS | ESS |
| --- | --- | --- |
| Number of APs | One | Multiple |
| Coverage | Small | Large |
| Roaming | Not supported | Supported |
| Example | Home Wi-Fi | Campus Wi-Fi |

### Conclusion

- BSS = Single Wi-Fi network (one AP)

- ESS = Multiple connected Wi-Fi networks (many APs, same SSID)

## 4. What are the basic functionalities of a Wi-Fi Access Point?

A Wi-Fi Access Point (AP) is a networking device that allows wireless devices to connect to a wired network.

It acts as a bridge between wireless clients and the internet.

### Main Functionalities of a Wi-Fi Access Point

### 1. SSID Broadcasting

- AP broadcasts the network name (SSID)

- Allows devices to discover and connect

Example: “Home_WiFi”, “College_Network”

### 2. Authentication and Security

- Verifies user identity before allowing access

- Supports security protocols like:

  - WPA2

  - WPA3

### 3. Data Transmission

- Transfers data between:

  - Wireless devices ↔ Access Point

  - Access Point ↔ Wired network

### 4. Medium Access Control (MAC Function)

- Controls how multiple devices use the wireless medium

- Uses CSMA/CA (Collision Avoidance)

Ensures no data collision happens

### 5. Signal Management

- Manages signal strength and coverage

- Adjusts transmission power

- Provides stable connectivity

### 6. Traffic Management

- Controls data flow between devices

- Prioritizes important traffic (QoS – Quality of Service)

Example: Video call gets priority over downloads

### 7. Roaming Support

- Helps devices switch between APs without disconnecting

- Used in ESS networks

### 8. Network Bridging

- Connects wireless network to wired LAN

- Acts as a bridge between two different network types

### Conclusion

A Wi-Fi Access Point performs multiple roles:

- Connects devices wirelessly

- Ensures secure communication

- Manages traffic and signals

- Provides seamless connectivity

## 5. Difference between Bridge Mode and Repeater Mode

In Wi-Fi networking, Bridge Mode and Repeater Mode are used to extend or connect networks, but they function differently.

### Bridge Mode

Bridge mode is used to connect two separate networks and make them act as a single network.

#### Key Features:

- Connects two LANs (wired or wireless)

- Works like a “bridge” between networks

- Does not create a new network

- Maintains original network performance

- Can use wired or wireless connection

#### How it Works:

- One router connects to another router

- The second router passes data without modifying it

#### Example:

- Connecting two buildings using wireless bridge

- Extending office network using Ethernet cable

### Repeater Mode

Repeater mode is used to extend the range of an existing Wi-Fi network.

#### Key Features:

- Receives signal and retransmits it

- Extends Wi-Fi coverage

- Works wirelessly

- Reduces speed due to signal repetition

#### How it Works:

- Repeater connects to main router

- Rebroadcasts the same Wi-Fi signal to farther areas

#### Example:

- Extending home Wi-Fi to another room or floor

### Difference between Bridge Mode and Repeater Mode

| Feature | Bridge Mode | Repeater Mode |
| --- | --- | --- |
| Purpose | Connect two networks | Extend Wi-Fi coverage |
| Connection Type | Wired/Wireless | Wireless only |
| Speed | High (no loss) | Reduced (signal duplication) |
| Network Creation | No new network | Extends existing network |
| Usage | Offices, buildings | Homes, small areas |

### Conclusion

- Bridge Mode is used for connecting networks without affecting speed

- Repeater Mode is used for increasing coverage but may reduce speed

## 6. Differences between 802.11a and 802.11b

The standards 802.11a and 802.11b are early Wi-Fi technologies defined by IEEE under the 802.11 family.

### 802.11a

#### Key Features:

- Operates in 5 GHz frequency band

- Maximum speed up to 54 Mbps

- Uses OFDM modulation

- Less interference (fewer devices use 5 GHz)

- Shorter range

#### Advantages:

- Faster speed

- Less congestion

#### Disadvantages:

- Lower coverage area

- Higher cost (earlier days)

### 802.11b

#### Key Features:

- Operates in 2.4 GHz frequency band

- Maximum speed up to 11 Mbps

- Uses DSSS (Direct Sequence Spread Spectrum)

- More interference (used by many devices)

- Longer range

#### Advantages:

- Better coverage

- Lower cost

#### Disadvantages:

- Slower speed

- More interference

### Comparison Table

| Feature | 802.11a | 802.11b |
| --- | --- | --- |
| Frequency | 5 GHz | 2.4 GHz |
| Maximum Speed | 54 Mbps | 11 Mbps |
| Range | Shorter | Longer |
| Interference | Less | More |
| Cost | Higher | Lower |

### Conclusion

- 802.11a - Faster, less interference, shorter range

- 802.11b - Slower, more interference, longer range

## 7. Configure your modem/hotspot to operate only in 2.4 GHz and 5 GHz, and compare the properties

This question involves a practical experiment where you test and compare Wi-Fi performance in 2.4 GHz and 5 GHz bands.

### Step 1: Configure 2.4 GHz Network

- Open mobile hotspot or router settings

- Select Wi-Fi band → 2.4 GHz only

- Turn ON hotspot/router

- Connect your laptop or mobile device

### Step 2: Note the Properties

Check Wi-Fi details in your device:

- Frequency band (2.4 GHz)

- Link speed (Mbps)

- Signal strength

- Network stability

### Step 3: Configure 5 GHz Network

- Change hotspot/router band to 5 GHz only

- Reconnect your device

- Observe the same parameters

### Observations

| Parameter | 2.4 GHz | 5 GHz |
| --- | --- | --- |
| Frequency | 2.4 GHz | 5 GHz |
| Speed | Lower | Higher |
| Range | Longer | Shorter |
| Wall Penetration | Better | Poor |
| Interference | High (many devices) | Low |
| Stability | Moderate | High (near router) |

### Explanation

- 2.4 GHz travels farther and passes through walls easily but suffers from interference (Bluetooth, microwave, etc.)

- 5 GHz provides higher speed and less interference but works best at shorter distances

### Conclusion

- Use 2.4 GHz for larger coverage areas

- Use 5 GHz for high-speed internet in shorter range

## 8. What is the difference between IEEE and Wi-Fi Alliance?

Wi-Fi technology is governed and standardized by two main organizations:

- IEEE

- Wi-Fi Alliance

### IEEE (Institute of Electrical and Electronics Engineers)

#### Role:

- Develops technical standards for Wi-Fi

- Defines the 802.11 protocols

#### Responsibilities:

- Specifies frequency bands, data rates, and modulation techniques

- Creates standards like:

  - 802.11a

  - 802.11b

  - 802.11n

  - 802.11ac

  - 802.11ax

It focuses on how Wi-Fi should work technically

### Wi-Fi Alliance (WFA)

#### Role:

- Certifies Wi-Fi devices for compatibility

#### Responsibilities:

- Tests devices from different companies

- Ensures devices work together properly

- Provides branding like:

  - Wi-Fi 4

  - Wi-Fi 5

  - Wi-Fi 6

It focuses on making sure devices work together in real life

### Difference between IEEE and Wi-Fi Alliance

| Feature | IEEE | Wi-Fi Alliance |
| --- | --- | --- |
| Type | Technical organization | Industry organization |
| Role | Creates standards | Certifies devices |
| Focus | Protocol design | Compatibility & branding |
| Example | 802.11ax | Wi-Fi 6 certification |

### Conclusion

- IEEE develops the technical foundation of Wi-Fi

- Wi-Fi Alliance ensures devices follow those standards and work together smoothly

## 9. What are the different types of Wi-Fi backhaul?

Backhaul refers to the connection that links a Wi-Fi network (like a router or access point) to the main internet source.

In simple terms, it is the path through which internet data reaches your Wi-Fi network.

### Types of Wi-Fi Backhaul

### 1. Wired Backhaul (Ethernet)

- Uses Ethernet cables to connect routers or access points

- Most reliable and stable connection

- Provides high speed with minimal latency

#### Advantages:

- No signal interference

- High performance

- Stable connection

#### Example:

- Fiber internet - Router - Access Point using Ethernet cable

### 2. Wireless Backhaul

- Uses wireless signals to connect routers or mesh nodes

- No cables required

- Common in mesh Wi-Fi systems

#### Advantages:

- Easy to install

- Flexible placement

#### Disadvantages:

- Speed may reduce due to signal sharing

- Affected by interference

#### Example:

- Mesh Wi-Fi system in homes or offices

### 3. Fiber Backhaul

- Uses optical fiber cables

- Provides extremely high speed and bandwidth

#### Advantages:

- Very high data rates

- Low latency

- Long-distance communication

#### Example:

- Internet Service Providers (ISPs) use fiber to deliver internet to homes

### 4. DSL Backhaul

- Uses telephone lines

- Slower compared to fiber and Ethernet

#### Advantages:

- Widely available

- Cost-effective

#### Disadvantages:

- Lower speed

- Higher latency

### Comparison Table

| Type | Medium | Speed | Reliability |
| --- | --- | --- | --- |
| Wired (Ethernet) | Cable | High | Very High |
| Wireless | Air | Medium | Moderate |
| Fiber | Optical Cable | Very High | Very High |
| DSL | Telephone Line | Low | Moderate |

### Conclusion

- Wired and Fiber backhaul - Best performance

- Wireless backhaul - Flexible but may reduce speed

- DSL - Basic connectivity with lower speed

## 10. Different Wi-Fi topologies and their use cases

Wi-Fi topology refers to the structure or arrangement of devices in a wireless network.

### 1. Infrastructure Topology

- Most common type

- Uses an Access Point (AP)

- Devices communicate through the AP

#### Features:

- Centralized control

- Stable connection

- Easy management

#### Use Case:

- Homes

- Offices

- Schools

### 2. Ad-hoc Topology (Peer-to-Peer)

- No Access Point required

- Devices connect directly to each other

#### Features:

- Simple setup

- Temporary connection

#### Use Case:

- File sharing between two laptops

- Small temporary networks

### 3. Mesh Topology

- Multiple APs (nodes) interconnected

- Each node communicates with others

#### Features:

- Wide coverage

- Self-healing network

- Seamless roaming

#### Use Case:

- Large homes

- Campuses

- Malls

### 4. Point-to-Point Topology

- Direct connection between two locations

#### Features:

- Long-distance communication

- High-speed link

#### Use Case:

- Connecting two buildings

- Industrial networks

### 5. Point-to-Multipoint Topology

- One central AP connects multiple devices

#### Features:

- One-to-many communication

- Efficient for distribution

#### Use Case:

- Internet service distribution

- Office networks

### Comparison Table

| Topology | Description | Use Case |
| --- | --- | --- |
| Infrastructure | AP-based network | Home, office |
| Ad-hoc | Device-to-device | Temporary sharing |
| Mesh | Multiple interconnected APs | Large areas |
| Point-to-Point | Two-node connection | Building links |
| Point-to-Multipoint | One-to-many connection | ISPs, offices |

### Conclusion

- Different topologies are used based on network size and requirement

- Infrastructure is most common

- Mesh is best for large coverage

- Ad-hoc is useful for temporary connections