Module 2

Assessment Questions:

Answers :

# 1. Brief about Split MAC Architecture and How It Improves AP Performance

## Introduction

In enterprise wireless networks, Access Points (APs) are usually lightweight devices managed by a centralized Wireless LAN Controller (WLC). This architecture is called Split MAC because the Medium Access Control (MAC) functions are divided between the AP and the controller.

## MAC Layer Responsibilities

The MAC layer handles wireless communication tasks such as:

- Frame transmission

- Acknowledgments

- Encryption and decryption

- Authentication

- Roaming

- Association management

In Split MAC, these responsibilities are divided intelligently.

## Functions Performed by AP

The AP handles all time-sensitive, real-time operations:

- Sending Beacon frames

- Responding to Probe Requests

- Frame acknowledgments (ACKs)

- Encryption and decryption

- Retransmissions

- RF signal transmission and reception

These tasks require immediate action and cannot tolerate network delay.

## Functions Performed by WLC

The WLC handles centralized tasks:

- User authentication

- Security policy enforcement

- VLAN assignment

- Roaming decisions

- QoS policies

- RF management

- Configuration management

## Why Split MAC is Needed

Wireless communication requires some decisions to be made within microseconds. If every decision had to travel to the controller, latency would increase significantly.

Split MAC ensures:

- Fast local decisions at AP

- Centralized intelligence at WLC

## Performance Improvements

### Reduced Processing Load

The AP focuses only on radio operations.

### Faster Response

Critical wireless frames are processed instantly.

### Better Scalability

A single controller can manage hundreds of APs.

### Simplified Management

Administrators configure policies once on the controller.

### Seamless Roaming

WLC coordinates client movement between APs.

## Real-World Example

When a client sends a data frame:

- AP immediately acknowledges the frame.

- AP encapsulates it in CAPWAP.

- WLC applies security and forwarding policies.

This division ensures both speed and centralized control.

# 2. Describe CAPWAP and Explain the Flow Between AP and Controller

## What is CAPWAP?

CAPWAP stands for:

Control And Provisioning of Wireless Access Points

It is the protocol used by lightweight APs to communicate with the Wireless LAN Controller.

## Purpose of CAPWAP

- AP discovery

- Controller join

- Configuration download

- Firmware updates

- Traffic tunneling

- Monitoring and management

## CAPWAP Ports

- UDP 5246 – Control Tunnel

- UDP 5247 – Data Tunnel

## Complete AP-to-WLC Flow

### Step 1: AP Bootup

- AP powers on.

- Performs POST.

- Loads operating system.

### Step 2: IP Address Acquisition

- AP obtains IP address via DHCP.

- Also receives subnet mask, gateway, and DNS.

### Step 3: WLC Discovery

Methods include:

- DHCP Option 43

- DNS query (CISCO-CAPWAP-CONTROLLER)

- Broadcast (same subnet)

- Previously saved controller IP

- Manual configuration

### Step 4: Controller Selection

- AP selects the best available WLC.

- Priority rules are applied.

### Step 5: DTLS Handshake

- AP and WLC authenticate each other using certificates.

- Secure tunnel is established.

### Step 6: Join Request

- AP sends Join Request.

- WLC validates AP credentials.

### Step 7: Configuration Download

- WLC sends:

  - WLAN configurations

  - SSIDs

  - Security settings

  - RF parameters

### Step 8: Firmware Verification

- If versions mismatch, WLC upgrades AP.

### Step 9: Tunnel Establishment

- Control tunnel created.

- Data tunnel created.

### Step 10: Operational State

- AP starts broadcasting SSIDs.

- Clients can now connect.

# 3. Where CAPWAP Fits in OSI Model, What Are the Two Tunnels and Their Purpose

## OSI Layer Placement

CAPWAP operates at:

- Layer 7 – Application Layer

It uses UDP at Layer 4.

## Transport Details

- UDP Port 5246 – Control

- UDP Port 5247 – Data

## Two CAPWAP Tunnels

### 1. Control Tunnel

#### Purpose

Carries management traffic between AP and WLC.

#### Information Transferred

- Join messages

- Configuration updates

- Keepalive packets

- Statistics

- Radio information

- Authentication exchanges

#### Security

- Protected using DTLS encryption.

### 2. Data Tunnel

#### Purpose

Carries actual client traffic.

#### Information Transferred

- Client data packets

- DHCP requests

- DNS traffic

- Internet traffic

#### Operation

- AP encapsulates wireless frames into CAPWAP.

- Sends them to WLC.

## Why Two Separate Tunnels?

- Better security.

- Simplified troubleshooting.

- Independent management.

- Efficient traffic handling.

# 4. Difference Between Lightweight APs and Cloud-Based APs

| Feature | Lightweight AP | Cloud-Based AP |
| --- | --- | --- |
| Management | Local WLC | Cloud Portal |
| Controller | On-Premises | Cloud Hosted |
| Deployment | Campus Networks | Branch Offices |
| Internet Dependency | Low | High |
| Scalability | Moderate | Excellent |
| Maintenance | Manual | Automated |

## Lightweight AP

### Characteristics

- Depends on WLC.

- Uses CAPWAP.

- Centralized control.

### Best For

- Large enterprise campuses.

- Low-latency environments.

## Cloud-Based AP

### Characteristics

- Managed through cloud dashboard.

- No physical controller required.

### Best For

- Retail chains.

- Remote offices.

- Distributed enterprises.

## Key Difference

- Lightweight AP uses on-prem controller.

- Cloud AP uses internet-hosted controller.

# 5. How CAPWAP Tunnel Is Maintained Between AP and Controller

## Keepalive Mechanism

AP and WLC periodically exchange:

- Echo Request

- Echo Response

This confirms connectivity.

## DTLS Session

- Security session remains active.

- Encryption keys are maintained.

## Sequence Number Monitoring

- Detects packet loss.

- Ensures proper packet order.

## Tunnel Health Monitoring

- Latency checks

- Packet loss checks

- Retransmission monitoring

## Failure Detection

If multiple keepalives are missed:

- Tunnel considered down.

- AP enters recovery mode.

## Reconnection Process

- AP restarts discovery.

- Rejoins primary or backup controller.

# 6. Difference Between Sniffer Mode and Monitor Mode, Use Cases

## Sniffer Mode

### Purpose

Captures wireless packets for analysis.

### Features

- Forwards captured packets to analyzer.

- Works with Wireshark.

- Detailed packet inspection.

### Use Cases

- Authentication failures

- Roaming issues

- Performance troubleshooting

- Protocol analysis

## Monitor Mode

### Purpose

Monitors RF environment continuously.

### Features

- Scans all channels.

- Detects threats.

- Identifies interference.

### Use Cases

- Rogue AP detection

- Wireless IDS/IPS

- Security monitoring

- Interference analysis

## Comparison Table

| Aspect | Sniffer | Monitor |
| --- | --- | --- |
| Packet Capture | Yes | Limited |
| Security Detection | Basic | Advanced |
| Wireshark Support | Yes | No |
| Client Service | No | No |

# 7. If WLC Is Deployed in WAN, Which AP Mode Is Best and How?

## Best Choice: FlexConnect

## Why FlexConnect?

- Local data switching.

- Centralized management.

- WAN bandwidth savings.

## Working

### Control Traffic

- Sent to central WLC.

### Client Data

- Switched locally at branch switch.

## Benefits

- WAN optimization.

- Better application performance.

- Survivability during WAN outage.

## WAN Failure Scenario

- Existing clients continue working.

- Local authentication possible.

- Local switching continues.

## Ideal Use Cases

- Branch offices

- Remote campuses

- Retail stores

# 8. Challenges of Deploying Autonomous APs in Large Networks

## Major Challenges

### 1. Manual Configuration

Each AP requires separate setup.

### 2. Inconsistent Policies

Security settings may vary.

### 3. Difficult Roaming

No centralized mobility management.

### 4. Firmware Management

Updates must be applied individually.

### 5. Complex Troubleshooting

No single management interface.

### 6. Scalability Issues

Managing 50+ APs becomes impractical.

### 7. RF Coordination Problems

Channel and power planning become difficult.

### 8. Security Risks

Higher chance of misconfiguration.

## Conclusion

Autonomous APs are suitable only for small deployments.

Large environments require controller-based architecture.

# 9. What Happens When WLC Goes Down for Lightweight AP in Local Mode?

## Immediate Effect

- CAPWAP tunnel disconnects.

## Existing Clients

- Usually lose connectivity.

- Data forwarding stops.

## New Clients

- Cannot join network.

- Authentication fails.

## AP Behavior

- Stops receiving controller instructions.

- Attempts to discover backup WLC.

## Recovery Options

### Primary/Secondary WLC

- AP joins alternate controller automatically.

### HA SSO

- Near-zero downtime.

## Important Note

In Local Mode:

- Both control and data traffic depend on WLC.

Therefore, controller failure directly impacts service.

## Exception

If using FlexConnect with local switching:

- Existing clients may continue working even if WLC becomes unreachable.