Module 4

Assessment Questions:

Answers :

# 1. Significance of MAC Layer and Its Position in the OSI Model

## Position in OSI Model

The MAC (Medium Access Control) layer is a sublayer of the Data Link Layer (Layer 2) in the OSI model.

Layer 7 - Application

Layer 6 - Presentation

Layer 5 - Session

Layer 4 - Transport

Layer 3 - Network

Layer 2 - Data Link

├── LLC

└── MAC

Layer 1 - Physical

## Significance of MAC Layer

The MAC layer is responsible for controlling how devices access the wireless medium.

### Primary Responsibilities

- Frame encapsulation

- Addressing using MAC addresses

- Error detection

- Medium access coordination

- Fragmentation and reassembly

- Retransmissions

- QoS implementation

- Security handling

## Why MAC is Important in WLAN

Unlike wired networks, wireless devices share the same radio medium.

The MAC layer ensures:

- Fair medium access

- Collision avoidance

- Efficient transmission

- Reliable delivery

- Secure communication

# 2. 802.11 MAC Header Frame Format and Field Purpose

## Standard MAC Frame Structure

## | Frame Control | Duration | Address 1 | Address 2 | Address 3 |

## | Sequence Control | Address 4 | QoS Control | HT Control | Frame Body | FCS |

## Detailed Field Explanation

### Frame Control (2 Bytes)

Contains:

- Protocol version

- Type

- Subtype

- To DS

- From DS

- More Fragments

- Retry

- Power Management

- More Data

- Protected Frame

- Order

### Duration/ID (2 Bytes)

- Reserves medium using NAV.

- Used for power-saving stations.

### Address Fields

- Address 1: Receiver Address

- Address 2: Transmitter Address

- Address 3: Destination or BSSID

- Address 4: Used in WDS only

### Sequence Control

- Sequence Number

- Fragment Number

Helps in ordering and duplicate detection.

### QoS Control

- Traffic priority

- TID

- EOSP

### HT Control

- Used in high-throughput operations.

# 3. MAC Layer Functionalities in Management, Control and Data Planes

## Management Plane

Responsible for network establishment and maintenance.

### Functions

- Beacon transmission

- Probe request/response

- Authentication

- Association

- Reassociation

- Deauthentication

- Disassociation

- Scanning

- Roaming

## Control Plane

Responsible for medium coordination.

### Functions

- RTS

- CTS

- ACK

- Block ACK

- PS-Poll

- CF-End

- BAR (Block ACK Request)

## Data Plane

Responsible for user traffic delivery.

### Functions

- Data transmission

- Frame aggregation

- Encryption

- Fragmentation

- Retransmission

- QoS scheduling

### Frame Check Sequence (FCS)

- CRC-based error detection.

# 4. Scanning Process and Its Types

## Purpose

Scanning allows clients to discover available wireless networks.

## Types of Scanning

### Passive Scanning

- Client listens for beacons.

- No transmission required.

#### Advantages

- Low overhead

- Standard compliant

#### Disadvantages

- Slower discovery

### Active Scanning

- Client sends Probe Request.

- AP responds with Probe Response.

#### Advantages

- Faster discovery

#### Disadvantages

- More airtime usage

## Scanning Steps

- Select channel

- Listen or transmit probe

- Receive responses

- Store AP information

- Move to next channel

- Choose best AP

# 5. Client Association Process

## Complete Workflow

### Step 1: Scanning

Client discovers APs.

### Step 2: Authentication

Open System Authentication.

### Step 3: Association Request

Client sends capabilities and supported rates.

### Step 4: Association Response

AP assigns Association ID (AID).

### Step 5: Security Handshake

EAPOL 4-way handshake occurs.

### Step 6: DHCP

Client obtains IP address.

# 6. EAPOL 4-Way Handshake and Key Purpose

## Purpose

- Verify PMK possession

- Generate session keys

- Install encryption keys

## Keys Involved

- PMK (Pairwise Master Key)

- PTK (Pairwise Transient Key)

- GTK (Group Temporal Key)

## Handshake Steps

### Message 1

AP sends ANonce.

### Message 2

Client generates SNonce, derives PTK, sends MIC.

### Message 3

AP verifies PTK, sends GTK encrypted.

### Message 4

Client acknowledges successful installation.

## PTK Components

- KCK: Key Confirmation Key

- KEK: Key Encryption Key

- TK: Temporal Key

# 7. Power Saving Schemes in MAC Layer

## Purpose

Reduce battery consumption.

## Legacy Power Save

- Client sleeps.

- AP buffers frames.

- Beacon indicates pending data.

- Client sends PS-Poll.

## U-APSD

- Automatic delivery upon trigger frame.

## TWT (Target Wake Time)

- Scheduled wake-up intervals.

- Major Wi-Fi 6 enhancement.

## Benefits

- Extended battery life

- Reduced contention

- Improved efficiency

# 8. Medium Access Control Methodologies

## CSMA/CA

Carrier Sense Multiple Access with Collision Avoidance.

## Working

- Sense channel.

- If idle, wait DIFS.

- Start random backoff.

- Transmit when timer reaches zero.

- Receive ACK.

## Optional RTS/CTS

Used to avoid hidden node problems.

## Why Not CSMA/CD?

Wireless devices cannot detect collisions while transmitting.

# 9. Block ACK Mechanism and Advantages

## Traditional ACK

Each frame acknowledged individually.

## Block ACK

Multiple frames acknowledged together.

## Process

- Sender transmits burst.

- Receiver sends bitmap acknowledgment.

## Advantages

- Reduced overhead

- Higher throughput

- Better efficiency

- Lower latency

# 10. A-MSDU, A-MPDU and A-MSDU within A-MPDU

## A-MSDU

### Definition

Multiple MSDUs combined into one MPDU.

### Advantages

- Reduced MAC header overhead.

### Limitation

- Entire aggregate retransmitted if corrupted.

## A-MPDU

### Definition

Multiple MPDUs combined into one transmission.

### Advantages

- Selective retransmission.

- Better reliability.

## A-MSDU within A-MPDU

### Hybrid Aggregation

- Multiple MSDUs inside each MPDU.

- Multiple MPDUs aggregated together.

## Comparison

| Feature | A-MSDU | A-MPDU |
| --- | --- | --- |
| Efficiency | Higher | High |
| Reliability | Lower | Higher |
| Retransmission | Entire Aggregate | Individual MPDU |

## Benefits of Hybrid Aggregation

- Maximum throughput

- Efficient airtime utilization

- Improved reliability

- Used extensively in modern Wi-Fi standards