Module 5

Assessment Questions:

Answers :

## 1) What are the key features of Wi-Fi 6, 6E, and 7, and how do they differ from Wi-Fi 5 (802.11ac)?

### Wi-Fi 5 (802.11ac)

Wi-Fi 5 mainly focused on increasing speed.

### Key Features

- Operates only in the 5 GHz band

- Supports MU-MIMO (downlink only)

- Maximum channel width: 160 MHz

- Maximum modulation: 256-QAM

- Suitable for HD streaming and normal office usage

### Limitations

- No support for 2.4 GHz

- Less efficient in crowded environments

- Higher latency when many devices connect simultaneously

### Wi-Fi 6 (802.11ax)

Wi-Fi 6 is designed for efficiency, capacity, and performance.

### Major Improvements

- Supports both 2.4 GHz and 5 GHz

- Introduces OFDMA

- Supports uplink and downlink MU-MIMO

- Uses 1024-QAM

- Includes Target Wake Time (TWT)

- Better battery life for connected devices

- Reduced latency

### Benefits

- Handles crowded networks efficiently

- Ideal for homes, offices, stadiums, and airports

### Wi-Fi 6E

Wi-Fi 6E is essentially Wi-Fi 6 extended into the 6 GHz band.

### Additional Features

- Supports 2.4 GHz, 5 GHz, and 6 GHz

- Access to many new non-overlapping channels

- Extremely low interference

- Lower latency

- Better for AR, VR, and gaming

### Wi-Fi 7 (802.11be)

Wi-Fi 7 is the next generation focused on ultra-high speed and ultra-low latency.

### New Innovations

- Supports 320 MHz channels

- Uses 4096-QAM

- Introduces Multi-Link Operation (MLO)

- Multi-RU allocation

- Preamble puncturing

- Extremely low latency

### Applications

- 8K streaming

- Cloud gaming

- AR/VR

- Industrial automation

### Comparison Table

| Feature | Wi-Fi 5 | Wi-Fi 6 | Wi-Fi 6E | Wi-Fi 7 |
| --- | --- | --- | --- | --- |
| Frequency Bands | 5 GHz | 2.4 & 5 GHz | 2.4, 5 & 6 GHz | 2.4, 5 & 6 GHz |
| Max QAM | 256 | 1024 | 1024 | 4096 |
| Channel Width | 160 MHz | 160 MHz | 160 MHz | 320 MHz |
| OFDMA | No | Yes | Yes | Enhanced |
| MLO | No | No | No | Yes |

## 2) Explain the role of OFDMA in Wi-Fi 6 and how it improves network efficiency.

### What is OFDMA?

OFDMA stands for Orthogonal Frequency Division Multiple Access.

It divides a Wi-Fi channel into smaller sub-channels called Resource Units (RUs).

### Traditional Wi-Fi

- One device transmits at a time.

- Other devices wait.

### Wi-Fi 6 with OFDMA

- Multiple devices can transmit simultaneously.

- Each device gets its own RU.

### Example

A 20 MHz channel can be divided among:

- Mobile phone

- Laptop

- Smart TV

- IoT sensor

All can communicate at once.

### Advantages

- Lower latency

- Better spectrum utilization

- Improved efficiency

- Reduced waiting time

- Excellent performance in crowded networks

### Real-World Scenario

In a classroom with 50 students, OFDMA allows many devices to send data simultaneously rather than one after another.

## 3) Discuss the benefits of Target Wake Time (TWT) in Wi-Fi 6 for IoT devices.

### What is TWT?

TWT stands for Target Wake Time.

It allows an access point and client device to schedule exactly when communication will occur.

### How It Works

- Device sleeps most of the time.

- Wakes up only at predetermined intervals.

- Exchanges data quickly.

- Returns to sleep.

### Benefits for IoT Devices

- Significant battery savings

- Longer device lifetime

- Reduced channel congestion

- Better power management

- Improved network scalability

### Example

A smart temperature sensor may wake every 10 minutes, send data, and sleep again.

### Applications

- Smart locks

- Sensors

- Wearables

- Smart meters

- Home automation devices

## 4) Explain the significance of the 6 GHz frequency band in Wi-Fi 6E.

### Why 6 GHz Matters

The 6 GHz band provides a completely new spectrum exclusively for modern Wi-Fi devices.

### Key Advantages

- More available bandwidth

- More non-overlapping channels

- Less interference

- Lower latency

- No legacy device congestion

### Technical Benefits

- Up to seven 160 MHz channels

- Excellent for high-bandwidth applications

### Applications

- VR/AR

- 4K/8K streaming

- Cloud gaming

- Video conferencing

### Important Note

Higher frequency means slightly shorter range compared to 5 GHz.

## 5) Compare Wi-Fi 6 and Wi-Fi 6E in terms of range, bandwidth, and interference.

| Parameter | Wi-Fi 6 | Wi-Fi 6E |
| --- | --- | --- |
| Range | Better | Slightly Lower |
| Bandwidth | High | Very High |
| Interference | Moderate | Very Low |
| Congestion | Possible | Minimal |
| Best Use | General Networking | Premium Applications |

### Detailed Explanation

#### Range

- Wi-Fi 6 on 2.4/5 GHz covers larger areas.

- Wi-Fi 6E on 6 GHz has shorter coverage.

#### Bandwidth

- Wi-Fi 6E offers much wider clean spectrum.

#### Interference

- Wi-Fi 6E experiences almost no interference due to the new band.

## 5) Compare Wi-Fi 6 and Wi-Fi 6E in terms of range, bandwidth, and interference.

| Parameter | Wi-Fi 6 | Wi-Fi 6E |
| --- | --- | --- |
| Range | Better | Slightly Lower |
| Bandwidth | High | Very High |
| Interference | Moderate | Very Low |
| Congestion | Possible | Minimal |
| Best Use | General Networking | Premium Applications |

### Detailed Explanation

#### Range

- Wi-Fi 6 on 2.4/5 GHz covers larger areas.

- Wi-Fi 6E on 6 GHz has shorter coverage.

#### Bandwidth

- Wi-Fi 6E offers much wider clean spectrum.

#### Interference

- Wi-Fi 6E experiences almost no interference due to the new band.

## 6) What are the major innovations introduced in Wi-Fi 7 (802.11be)?

### Key Innovations

#### 1. Multi-Link Operation (MLO)

Uses multiple bands simultaneously.

#### 2. 320 MHz Channels

Doubles channel width.

#### 3. 4096-QAM

Improves data density.

#### 4. Multi-RU

Efficient resource allocation.

#### 5. Preamble Puncturing

Uses partially blocked channels effectively.

### Benefits

- Up to 46 Gbps theoretical speed

- Ultra-low latency

- Higher reliability

- Better performance in congested environments

## 7) Explain Multi-Link Operation (MLO) and its impact on throughput and latency.

### What is MLO?

MLO allows a device to use multiple wireless links simultaneously.

For example:

- 5 GHz + 6 GHz together

### How It Works

- Data packets are split across multiple bands.

- Traffic can dynamically switch between links.

### Impact on Throughput

- Aggregate bandwidth increases significantly.

- Faster downloads and uploads.

### Impact on Latency

- Packets take the fastest available path.

- Reduces delay and jitter.

### Reliability

If one link degrades, traffic instantly shifts to another.

### Example

Gaming laptop using both 5 GHz and 6 GHz simultaneously.

## 8) What is the purpose of 802.11k and 802.11v, and how does it aid roaming?

### 802.11k – Neighbor Reports

- AP provides a list of nearby access points.

- Client knows where to roam next.

### 802.11v – Network Assisted Roaming

- Network suggests the best AP.

- Helps clients make smarter roaming decisions.

### Benefits

- Faster roaming

- Better signal quality

- Reduced scanning time

- Improved user experience

### Example

While walking through an office, your phone receives guidance on the next AP.

## 9) Explain Fast BSS Transition (802.11r) and its benefit in mobile environments.

### What is 802.11r?

802.11r enables fast roaming by reducing authentication time.

### Traditional Roaming

- Full authentication occurs every time.

- Causes delay.

### With 802.11r

- Security keys are pre-shared.

- Handoff occurs quickly.

### Benefits

- Roaming delay reduced to less than 50 ms

- Seamless voice calls

- Smooth video streaming

- No packet loss

### Applications

- VoIP

- Video conferencing

- Warehouse scanners

- Mobile healthcare devices

## 10) How do 802.11k/v/r work together to provide seamless roaming?

### Combined Operation

#### Step 1: 802.11k

Provides neighboring AP information.

#### Step 2: 802.11v

Suggests the optimal AP.

#### Step 3: 802.11r

Performs rapid secure handoff.

### Roaming Process

- Device signal weakens.

- 802.11k identifies nearby APs.

- 802.11v recommends best AP.

- 802.11r completes fast authentication.

- Connection continues uninterrupted.

### Benefits

- Near-zero roaming delay

- Better voice quality

- Uninterrupted video calls

- Improved mobility

- Enterprise-grade wireless experience

### Real-World Example

A doctor moving across a hospital while on a VoWiFi call experiences no call drop.