Module 6

Assessment Questions:

Answers :

# 1) What are the pillars of Wi-Fi security?

Wi-Fi security is built upon three fundamental pillars:

## A. Authentication

Authentication verifies the identity of a user or device before granting network access.

### Purpose

- Ensures only authorized users can connect.

- Prevents unauthorized access.

### Example

- Entering a Wi-Fi password.

- Using enterprise credentials via 802.1X.

## B. Encryption

Encryption protects data transmitted over the air.

### Purpose

- Converts readable data into unreadable ciphertext.

- Prevents eavesdropping.

### Example

- AES encryption in WPA2/WPA3.

## C. Integrity

Integrity ensures data is not altered during transmission.

### Purpose

- Detects tampering.

- Prevents packet modification attacks.

### Example

- Message Integrity Check (MIC).

## Additional Security Components

- Access Control

- Key Management

- Non-repudiation

- Availability

# 2) Explain the difference between authentication and encryption in Wi-Fi security

| Feature | Authentication | Encryption |
| --- | --- | --- |
| Purpose | Verifies identity | Protects data |
| When Used | Before access | During communication |
| Goal | Access control | Confidentiality |
| Example | Password, 802.1X | AES, GCMP |

## Simple Analogy

- Authentication is like showing your ID at a building entrance.

- Encryption is like speaking in a secret language once inside.

## Process Flow

- Device requests connection.

- Authentication verifies credentials.

- Encryption secures subsequent traffic.

# 3) Explain the differences between WEP, WPA, WPA2, and WPA3

| Standard | Year | Encryption | Security Level |
| --- | --- | --- | --- |
| WEP | 1997 | RC4 | Very Weak |
| WPA | 2003 | TKIP | Moderate |
| WPA2 | 2004 | AES-CCMP | Strong |
| WPA3 | 2018 | SAE + GCMP | Very Strong |

## WEP

- First Wi-Fi security standard.

- Uses static keys.

- Easily crackable.

## WPA

- Temporary replacement for WEP.

- Introduced TKIP.

- Improved but outdated.

## WPA2

- Mandatory standard for many years.

- Uses AES.

- Highly secure.

## WPA3

- Latest generation.

- Uses SAE handshake.

- Resistant to offline dictionary attacks.

# 4) Why is WEP considered insecure compared to WPA2 or WPA3?

## Major Weaknesses

### Short Initialization Vector

- Only 24 bits.

- Repeats frequently.

### Static Keys

- Same key used repeatedly.

### Weak RC4 Implementation

- Vulnerable to cryptographic attacks.

### Easy Cracking

- Can be broken within minutes.

## Attack Process

- Capture packets.

- Analyze IV repetitions.

- Recover the key.

## Why WPA2/WPA3 Are Better

- Dynamic session keys.

- Strong encryption.

- Secure handshakes.

# 5) Why was WPA2 introduced?

WPA2 was introduced to replace WPA and provide stronger security.

## Reasons

- WPA used TKIP, which had limitations.

- Increasing attack sophistication.

- Need for IEEE 802.11i compliance.

## Improvements

- AES-CCMP encryption.

- Better integrity protection.

- Stronger key management.

- Enterprise-grade security.

## Benefits

- Higher confidentiality

- Better integrity

- Long-term security

# 6) What is the role of the Pairwise Master Key (PMK) in the 4-way handshake?

## Definition

PMK is the master secret shared between the client and the access point.

## How PMK is Generated

### WPA2-Personal

- Derived from SSID and passphrase using PBKDF2.

### WPA2-Enterprise

- Generated during 802.1X authentication.

## Purpose

- Used to derive temporary encryption keys.

- Never transmitted over the air.

## Derived Keys

- PTK (Pairwise Transient Key)

- GTK (Group Temporal Key)

## Security Advantage

Even if session keys are compromised, the PMK remains protected.

# 7) How does the 4-way handshake ensure mutual authentication between the client and the access point?

## Step 1: AP Sends ANonce

- Access Point sends a random number.

## Step 2: Client Generates PTK

- Uses:

  - PMK

  - ANonce

  - SNonce

  - MAC addresses

Client sends SNonce and MIC.

## Step 3: AP Verifies

- AP independently computes PTK.

- Verifies client's MIC.

## Step 4: AP Sends GTK

- Encrypted using PTK.

- Includes MIC.

## Step 5: Client Verifies

- Validates AP's MIC.

## Result

Both sides prove they possess the same PMK.

# 8) What happens if we enter the wrong passphrase during a 4-way handshake?

## Process

- Client derives an incorrect PMK.

- Generates incorrect PTK.

- Computes invalid MIC.

## AP Verification

- AP compares received MIC.

- Verification fails.

## Outcome

- Handshake terminates.

- Authentication fails.

- Client cannot connect.

## User Experience

Typical messages include:

- "Incorrect password"

- "Unable to join network"

# 9) What problem does 802.1X solve in a network?

802.1X solves the problem of weak shared-password authentication.

## Problems with Pre-Shared Keys

- One password for all users.

- Difficult revocation.

- Poor accountability.

- Security risks when employees leave.

## 802.1X Solution

- Individual user authentication.

- Centralized policy enforcement.

- Dynamic key generation.

- Detailed logging.

## Components

- Supplicant (Client)

- Authenticator (AP/Switch)

- Authentication Server (RADIUS)

# 10) How does 802.1X enhance security over wireless networks?

## Strong Authentication

- Unique credentials per user.

## Centralized Management

- Authentication handled by RADIUS.

## Dynamic Encryption Keys

- Fresh keys for every session.

## Access Control

- Role-based permissions.

- VLAN assignment.

## Accounting and Auditing

- Complete login records.

## Security Benefits

- Prevents password sharing.

- Simplifies user revocation.

- Supports certificates.

- Ideal for enterprise networks.

# Summary Table

| Feature | WPA2-PSK | WPA2-Enterprise (802.1X) |
| --- | --- | --- |
| Authentication | Shared Password | Individual Credentials |
| Scalability | Small Networks | Large Enterprises |
| Security | Good | Excellent |
| User Tracking | No | Yes |